Experiment / Assignment / Tutorial No. 2

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

Batch: B1 Roll No.: 1711072 Experiment / assignment / tutorial No.: 2

Title: Binary Adders and Subtractors

_____________________________________________________________________________________

Objective: To implement half and full adder–subtractor using gates and IC 7483

______________________________________________________________________________

Expected Outcome of Experiment:

CO2: Use different minimization technique and solve combinational circuits, synchronous & asynchronous sequential circuits.

______________________________________________________________________________

Books/ Journals/ Websites referred:

- R. P. Jain, “Modern Digital Electronics”, Tata McGraw Hill

- M .Morris Mano, “Digital Logic & computer Design”, PHI

- http://physics.niser.ac.in/labmanuals/sem5/elect/7_ADDER%20SUBTRACTO  R%20CIRCUITS.pdf

Pre Lab/ Prior Concepts:

Adder: Addition of two binary digits is most basic operation performed by the digital computer. There are two types of adder:

- Half adder

- Full adder

Half Adder: Half adder is combinational logic circuit with two inputs and two outputs. It is the basic building block for addition of two single bit numbers.

Full adder: A half adder has a provision not to add a carry coming from the lower order bits when multi bit addition is performed. for this purpose a third input terminal is added and this circuits is to add A,B,C where A and B are the nth order bits of the number A and B respectively and C is the carry generated from the addition of (n-1) order bits. This circuit is referred to as full adder.

Subtractor: Subtraction of two binary digits is one of the most basic operations performed by digital computer .there are two types of subtractor:

- Half subtractor

- Full subtractor

Half subtractor: Logic circuit for the subtraction of B from A where A,B are 1 bit numbers is referred to as half subtract or .the subtract or process has two input and difference and borrow are the two outputs.

Full subtractor: As in the case of the addition using logic gates, a full subtractor is made by combining two half-sub tractors and an additional OR-gate. A full subtractor has the borrow in capability (denoted as BORIN) and so allows cascading which results in the possibility of multi-bit subtraction.

IC 7483

For subtraction of one binary number from another, we do so by adding 2’s complement of the former to the latter number using a full adder circuit.

IC 7483 is a 16 pin, 4-bit full adder. This IC has a provision to add the carry output to transfer and end around carry output using Co and C4 respectively.

2’s complement: 2’s complement of any binary no. can be obtained by adding 1 in 1’scomplement of that no.

e.g.  2’s complement of +(10)10 =1010is

| 1C of 1010 |  |  | 0101 |
| --- | --- | --- | --- |
|  |  | + | 1 |
| -(10)10 |  |  | 0110 |

In 2’s complement subtraction using IC 7483, we are representing negative number in 2’s complement form and then adding it with 1st number.

Implementation Details:

Half Adder Block Diagram

Half Adder Circuit

Truth Table for Half Adder

| Inputs |  | Outputs |  |
| --- | --- | --- | --- |
| A | B | S | C |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

From the truth table (with steps):

S=A.B’+A’.B

C=A.B

Full Adder Block Diagram

Full Adder Circuit

Truth Table for Full Adder

| A | B | C | Sum | Carry |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

From the truth table (with steps):

S=A’.B’.Cin+A’.B.Cin’+A.B’.Cin’+A.B.Cin

Cout(Carry) =A.Cin+A.B+B.Cin

Half Subtractor Block Diagram

Half Subtractor Circuit

Truth Table for Half Subtractor

| A | B | DIFFERENCE(D) | BORROW(Bo) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

From the truth table (with steps) :

Difference(D)=A’.B+A.B’

Borrow(Bo)=A’.B

Full Subtractor Block Diagram

Full Subtractor Circuit

Truth Table for Full subtractor:

| A | B | BIN | D | BOROUT |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
|  |  |  |  | 1 |
| 1 | 1 | 1 | 1 | 1 |

From the truth table (with steps):

Difference (D)=A’.B’.Bin+A’.B.Bin’+A.B’.Bin’+A.B.Bin

BORout=A’.B+A’.Bin+B.Bin

IC 7483

Procedure:

- Locate the IC 7483 and 4-not gates block on trainer kit.

- Connect 1st input no. to A4-A1 input slot and 2nd (negative) no. to B4-B1 through 4-not gates (1C of 2nd no.)

- Connect high input to C0 so that it will get added with 1C of 2nd no. to get 2C.

- Connect 4-bit output to the output indicators.

- Switch ON the power supply and monitor the output for various input combinations.

Example:

| 1) 710 -210 = 510 |  |  |
| --- | --- | --- |
| 7 |  | 0111 |
| 2 |  | 0010 |
| 1’C of 2 |  | 1101 |
|  |  | + 1 |
| 2’C of 2 |  | 1110 |

0111

+ 1110  0101

Pin Diagram IC7483

Adder

Subtractor

Post Lab Descriptive Questions:

- What is difference between half and full adder, half and full subtractor?

Ans. Full adder and half adder:

The difference between half adder and full adder is that full adder has three inputs and two outputs. The two inputs are A and Band the third is a carry input Cin.The output carry is designated as Cout and the normal output is designated as S whereas half adder has two inputs and one output and carry is neglected in half adder. It isn’t neglected in full adder.

Full subtractor and half adder:

The difference between half subtractor and half subtractor is also the same. Half subtractor has two inputs and one output while full subtractor has 3 inputs and 2 outputs and bits are borrowed in full subtractor. It is not restricted to one bit subtraction.

- Perform the following Binary subtraction with the help of appropriate ICs:

- 7-5

- 5-7

- 9-4

Ans.  a) The binary form of 7 is:0111

The binary form of 5 is: 0101

1’s complement of 5 is:1010

+ 1

2’s complement of 5 is:1011

Thus 7-5 is:

0111

+1011

(0010)2 = (2)10

b) The binary form of 5 is: 0101

The binary form of 7 is: 0111

1’s complement of 7 is: 1000

+ 1

2’s complement of 7 is: 1001

Thus 5-7 is:

0101

+1001

(1110)2

The answer is a negative number. Hence we take the 2’s complement

1’s complement: 0001

+1

2’s complement: (0010)2= (2)10

- The binary form of 9 is:1001

The binary form of 4 is: 0100

1’s complement of 4 is: 1011

+ 1

2’s complement of 4 is:  1100

Thus 9-4 is:

1001

+1100

(0101)2 = (5)10

Conclusion:

The binary circuit for half/full adder and subtractor were studied by both basic gates and dedicated gates for various test cases with correct outputs.